Canadian Elevator Contractors Association Maintenance Control Program Log

Escalator & Moving Walk Devices

A Plan and Procedure Document to Maintain Equipment in Compliance with 8.6

www.ceca-acea.org

| TSSA Installation Number: | Registered Device Address: | Registered Contractor: |
| --- | --- | --- |
| 12345678 | 123 Anyroad Drive, Anytown, ON M8M 8M8 | ABC Elevator Maintenance Company |
| Device Identification: |  | Contractor Registration Number: |
| ESCALATOR #30 |  | 12345678 |
| Device Type: (i.e. escalator, moving walk) | - Per A17.1/B44 the maintenance frequency shall be based on equipment age, condition, wear, design qualities, usage, environment and or technology to effectively maintain equipment in compliance with the code of installation or alteration. Category 1, 3 and 5 testing is required to be performed at 1, 3 and 5 year intervals. The AHJ may impose frequencies other than those prescribed or recommended by A17.1/B44 or the Manufacturer. - AHJ prescribed frequencies are noted in the attached log sheets. - In Ontario, maintenance visit shall not extend beyond 3 months. |  |
| Escalator |  |  |
| RECORD OF MAINTENANCE VISIT |  |  |

| Year | J | F | M | A | M | J | J | A | S | O | N | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2014 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2015 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2016 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2017 |  |  |  |  |  |  |  |  |  |  |  |  |

* Denotes requirements from A17.1/B44 – 2013 (Optional until adoption of 2013 code)

Note: Mechanics are required to initial tasks once complete and compliant. Device Type(s): Escalator (ESC), Moving Walk (MW) Frequency: In Months. ‘A’ denotes Allowable, either Mandated (M) or Recommended(R). ‘I’ denotes Implemented on this device Frequency: Shall be indicated on the Log Book in the implemented column at the time the MCP and Log Book are initiated.

| ESCALATOR Maintenance Tasks | Device Type | Freq (Months) |  | MECHANIC TO INITIAL WHEN TASK IS COMPLETE AND COMPLIANT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | I | J | F | M | A | M | J | J | A | S | O | N | D |
| 8.6.8.1: Handrails | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.2: Step-to-Skirt Clearance | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.3: Step/Skirt Performance Index | ESC | 12M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.4: Combplates | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.5: Escalator Skirt Panels and Skirt Obstruction Devices | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.6: Steps | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.7: Rollers, Tracks, and Chains | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.8: Signs | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.9: Guards at Ceiling Intersections | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.10: Antislide Devices | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.11: Handrail Guards | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.12: Brakes | ESC | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.13: Cleaning | ESC | 12M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.14: Entrance and Egress Ends | ESC | 12M |  |  |  |  |  |  |  |  |  |  |  |  |  |

| MOVING WALKS Maintenance Tasks | Device Type | Freq (Months) |  | MECHANIC TO INITIAL WHEN TASK IS COMPLETE AND COMPLIANT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | I | J | F | M | A | M | J | J | A | S | O | N | D |
| 8.6.9.1: Handrails | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.8.2: Step-to-Skirt Clearance | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.2: Combplates | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.3: Pallets | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.2: Combplates | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.5: Belt-Type Treadway | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.6: Signs | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.8: Antislide Devices | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.9: Handrail Guards | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.10: Brakes | MW | 3M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.11: Cleaning | MW | 12R |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6.9.13: Clearances | MW | 12R |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | CAT 1 Test Requirements Escalators | A17.2 Item | Date Last Completed | Date | Completed by (Sign or Initial) | Deficiencies | Corrective Action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.6.8.15.1 | Machine Space Access. | 8.1.2 |  |  |  |  |  |
| 8.6.8.15.2 | Machine Space Stop Switch. | 8.2.2 |  |  |  |  |  |
| 8.6.8.15.3 | Controller and Fuses. | 8.3.2 |  |  |  |  |  |
| 8.6.8.15.4 | Driving machine and brake torque tests. | 8.4.2 |  |  |  |  |  |
| 8.6.8.15.5 | Speed Governor. | 8.3.2 |  |  |  |  |  |
| 8.6.8.15.6 | Broken drive chain device. | 8.6.2 |  |  |  |  |  |
| 8.6.8.15.7 | Reversal stop device. | 8.7.2 |  |  |  |  |  |
| 8.6.8.15.8 | Broken step chain device. | 8.8.2 |  |  |  |  |  |
| 8.6.8.15.9 | Step up thrust device. | 7.9.2 8.9.2 |  |  |  |  |  |
| 8.6.8.15.10 | Missing step device. | 10.9.2 |  |  |  |  |  |
| 8.6.8.15.11 | Step level device. detectors chainsafety system0000000000000000000000000000000000000000000000000000000000000000000000000000000000 | 8.11.2 |  |  |  |  |  |
| 8.6.8.15.12 | Step, chain and trusses. | 8.12.2 |  |  |  |  |  |
| 8.6.8.15.13 | Handrail and handrail monitoring safety systems. | 8.13.2 7.3.2 |  |  |  |  |  |
| 8.6.8.15.14 | Heaters and outdoor units. | 7.18.4 |  |  |  |  |  |
| 8.6.8.15.15 | Permissible stretch of step chain. | 8.12.2 |  |  |  |  |  |
| 8.6.8.15.16 | Disconnected motor device. | 8.6.2 |  |  |  |  |  |
| 8.6.8.15.17 | Response to smoke detectors. | 8.15.2 |  |  |  |  |  |
| 8.6.8.15.18 | Comb/ Step Impact Device. | 7.7.2 |  |  |  |  |  |
| 8.6.8.15.19 | Step/Skirt Performance Index. | 7.17.2 |  |  |  |  |  |
| 8.6.8.15.20 | Clearance Between Step and Skirt. | 7.7.2 |  |  |  |  |  |
| 8.6.8.15.21 | Inspection Controls. | 6.1.6.2.2 |  |  |  |  |  |
| 8.6.8.15.22 | Step lateral displacement device. | 7.9.2 |  |  |  |  |  |
| 8.6.8.15.23 | Seismic Risk Zones 2 or Greater | 7.20.2 |  |  |  |  |  |

Notes:

Category 1 tests must be conducted each calendar year and are due on the anniversary month of the previous category 1 test.

|  | CAT 1 Test Requirements Moving Walks | A17.2 Item | Date Last Completed | Date | Completed by (Sign or Initial) | Deficiencies | Corrective Action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.6.8.15.1 | Machine Space | 10.1.2 |  |  |  |  |  |
| 8.6.8.15.2 | Stop Switch | 10.2.2 |  |  |  |  |  |
| 8.6.8.15.3 | Controller and Wiring | 10.3.2 |  |  |  |  |  |
| 8.6.8.15.5 | Speed Governor | 10.5.2 |  |  |  |  |  |
| 8.6.8.15.6 | Broken Chain Device | 10.6.2 10.7.2 |  |  |  |  |  |
| 8.6.8.15.8 | Broken Step Chain on Treadway Device | 10.8.2 |  |  |  |  |  |
| 8.6.8.15.9 | Step Upthrust Device | 7.9.2 8.9.2 |  |  |  |  |  |
| 8.6.8.15.10 | Missing Step or Pallet Device | 10.10.2 |  |  |  |  |  |
| 8.6.8.15.11 | Step or Pallet Leveling Device | 10.11.2 |  |  |  |  |  |
| 8.6.8.15.11 | Step or Pallet Leveling Device | 10.13.2 9.3.2 |  |  |  |  |  |
| 8.6.8.15.12 | Steps, Pallet, Step or Pallet Chain, and Trusses | 10.12.2 |  |  |  |  |  |
| 8.6.8.15.13 | Handrail Safety Systems | 10.11.2 |  |  |  |  |  |
| 8.6.8.15.14 |  | 9.18.2 |  |  |  |  |  |
| 8.6.8.15.15 | Permisible Stretch in Escalator Chains | 10.12.2 |  |  |  |  |  |
| 8.6.8.15.16 | Disconnected Motor Safety Device | 10.6.2 |  |  |  |  |  |
| 8.6.8.15.17 | Response to Smoke Detectors | 10.15.2 |  |  |  |  |  |
| 8.6.8.15.18 | Comb Step or Comb Pallet Impact Device | 9.7.2 |  |  |  |  |  |
| 8.6.8.15.21 |  | 6.2.6.2.2 |  |  |  |  |  |
| 8.6.8.15.23 | Seismic Risk Zones 2 or Greater | 9.20.2 |  |  |  |  |  |

Notes:

Category 1 tests must be conducted each calendar year and are due on the anniversary month of the previous category 1 test.

| Repair and Replacement Records for Escalators and Moving Walks Note: All non-maintenance repairs or replacements must be recorded per 8.6.1.4.1 (b) Recording Keeping shall include but not limited to the following: Where a defective part directly affecting the safe operation is identified or any components in Category 1 |  |  |
| --- | --- | --- |
| Dates Performed | Corrective Action | Initials |
| Other Records for Escalators or Moving Walks not previously covered |  |  |

Mechanic Signature Legend: Certificate Numbers for all Mechanics must be listed.

| Mechanics Printed Name | EDM Certificate Number | Signature | Initials | Mechanics Printed Name | EDM Certificate Number | Signature | Initials |
| --- | --- | --- | --- | --- | --- | --- | --- |